Investigation of Inlet and Diffuser Geometry Modifications on Film Cooling Performance of Additively Manufactured Shaped Holes in Crossflow

Doctoral Dissertation Proposal

Fraser B. Jones III

Department of Mechanical Engineering

The University of Texas at Austin

November 2018

Objectives

To develop a better understanding of the relative importance of inlet and diffuser geometries on the performance of shaped holes in film cooling applications. To use these insights to guide the development of more effective film cooling holes using additive manufacturing.

- Investigate the effects of separation regions at the inlet of shaped holes.

It is well understood that a separation region exists at the inlet of traditionally manufactured shaped holes. My computational fluid dynamics (CFD) work shows that contouring and rounding of a shaped hole inlet could reduce inlet flow separation leading to a slower and more uniform flow entering the diffuser. Subsequent experiments on standard 7-7-7 shaped holes, additively manufactured (AM) with a 25% inlet fillet, show greatly improved performance in crossflow performance over a wide range of velocity ratios (VR) and channel velocity ratios (VRc). I propose a series of CFD and experiments designed to isolate and quantify the effects of inlet separation on industry relevant shaped hole geometries.

- Investigate the effects of metering hole length on shaped hole performance.

Shaped hole metering length plays an important part in mitigating the effects of separation before the flow enters the diffuser. Boundary Layer (BL) growth and turbulent mixing will cause the separation region to reduce in size and the velocity profile to become more uniform. However, fully developed flow would not be expected in the 1.5~6.0D metering hole length typical of shaped holes. The effects of metering hole length on shaped hole performance are poorly understood. I propose a series of CFD and experiments that vary metering hole length and compared the effects against my previous experiments to assess if metering hole length can be an effective tool in reducing the negative effects of inlet separation.

- Develop a better inlet region and matching diffuser geometry

Given the insight gained from the above experiments it should be much simpler to determine the relative impact on performance caused by flow separation at various locations within the film cooling hole. This could help tailor the development of a new film cooling hole that works more efficiently at wider operating condition ranges. I propose the creation of a new shaped hole that leverages AM to better utilize the diffuser in existing shaped holes over a wide range of operating conditions.

The final product of this dissertation will be a stronger understanding of inlet region effects on shaped hole performance and the creation of a hole geometry that leverages this information to increase performance over a wide range of operating conditions.

Gas-turbine energy production is a key component of modern aviation and power production. In 2017 alone, natural gas fired turbo machine power plants account for 26% of the United States energy production (cite EIA 2017) and completely dominate the aviation engine market. Engine manufactures are locked in fierce competition for market share as customers demand ever more efficient gas-turbine technology that reduce costs and emissions without sacrificing performance. All modern gas turbine engines follow the thermodynamics of the Brayton Power Cycle which requires increasing pressure or temperature ratios to improve thermal efficiency (cite a thermo book). Since combustion temperatures can easily exceed the melting point of even the best super alloys, the maximum operating temperature of an engine is limited by the ability to protect and cool hot end components from the intense heat. Up to 30% of the compressors airflow must be redirected from producing useful work and instead used to provide cooling for hot end components. This can become a limiting factor when increasing combustion temperature requires so much additional cooling that no net increase in efficiency may be achieved. Therefore, for any jet engine that requires cooling, the maximum thermal efficiency of an engine becomes directly limited by the efficiency of coolant usage.

The effective use of coolant can be split into two broad categories; internal and external heat transfer. The objective of the cooling schemes is to keep each components below its maximum operating temperature. Air from the compressor is extracted at temperatures and pressures of up to 60atm and 800~900ºC and used to cool the combustor and turbine components. While the coolant temperature remains below the components maximum operating temperature, typically around ~1100ºC (cite Bogard) it can be used as a heat sink by forced convection heat transfer inside complex internal cavities. At some point the coolant will have warmed so much that is no longer provides a meaningful convective heat sink but still far below the external gas temperature which can be in excess of 1400~1600ºC. By ejecting the coolant onto the surface of a component, a thin film of air can be created that buffers the surface from the hot working gas and lowers the convective driving temperature difference at the surface. We call this form of protection “film cooling” and it can provide up to [Need % from somewhere] of the overall cooling to a hot end component.

The success of failure of film cooling techniques is intricately linked to the hole geometry used to deliver coolant from internal passages to a components surface. The ideal geometry for delivering coolant to a surface is widely considered to be a slot which would provide the entire surface with a thin, coherent buffer of coolant [Cite Goldstein’s early work on slots]. Unfortunately, mechanical limitations of hot end components make slots impractical, therefore discreet film cooling holes are used as a compromise between thermal and mechanical constraints. The first discreet film cooling holes were simply rows of holes drilled normal to the components surface into large internal cavities of coolant. These cylindrical holes provided some measure of thermal protection but tend to eject the coolant far from the surface at high blowing rates [Need a citation from Goldstein or others]. Goldstein laid the foundation for modern “shaped” film cooling hole geometries by inclining the hole relative to the mainstream and expanding the exit of a cylindrical hole [cite Goldstein 1970’s]. These changes greatly reduced the coolant momentum and expanded the region covered by coolant resulting in significantly better performance over a wider range of operating conditions.

Unfortunately, film cooling is not as easy as finding the single best shaped hole geometry. A vast number of parameters effect the performance of shaped holes and physical limitations of manufacturing or component design may disallow many geometries. Most research is directed at understanding the complex physics of film cooling and creating shaped hole geometries that perform well over a large range of operating conditions.

[Cite Bogard’s Book Chapter]

Bogard and Thole [cite] have an excellent overview of the current research in film cooling physics identifying key parameters in film cooling. They distill the relevant research of all the factors effecting film cooling from [Table 1] except for “Shape of the hole”  where only Saumweber et al. is presented characterizing the effects of hole shaping against a cylindrical hole and no information on the effects of (l/d) is presented. In fact, papers that isolate the effects of hole geometry are extremely rare given the time and expense of manufacturing numerous test pieces for experiment. Gritsch [cite] experimentally investigate a wide variety of hole geometry parameters including L/D, P/D, C/P, AR, and Compound Angle. Their paper sets the standard for isolating and characterizing geometric effects on shaped holes but leaves some blind spots in hole geometry effects. While they tested a number of expansion angles and area ratios, their tests of L/D only looked at holes of at least 7.5, matching forward and lateral expansion angles of 4º, maximum lateral expansion angle of 7º, and AR were only tested between 2.5 and 4.7. These geometric limitations mean shorter holes with larger expansion angles are not well modeled by the Colban correlation. As holes become shorter and diffusers become wider, the effects of inlet separation regions are poorly understood. Only a few papers numerically investigate the inlet region and even fewer attempt to back up their findings with experiments.

Hyams [30] did a computational and experimental study on slot film cooling using RANS with standard k-ε (SKE) modeling to identify the inlet separation region as a cause of in hole turbulence generation and explore rounding of the inlet to qualify its effects. Hyams [31] also published a paper in 2000 that investigated entrance and in hole effects within cylindrical, shaped, cusp, and inlet shaped holes and concluded that RANS proved useful in comparing relative performance of shaped film cooling holes and giving logical reasons for why performance might increase or decrease. Unfortunately, both papers put a strong focus on turbulent effects but suffer from standard k-ε modeling being particularly poor at modeling turbulence generated by separation regions. Additionally, both papers only use plenum fed configurations and the second paper studies very complex geometry but only validates against cylindrical holes

Perhaps the best early paper looking at in hole effects come from Kohli and Thole [28]. They numerically investigate the entrance region of 15-15-15 shaped holes in plenum, crossflow, and counterflow configurations. Their streamline results show the strong swirling motion induced by the crossflow while the vector cutplanes show a separation region which can strongly affect the performance of a hole. Unfortunately, their experimental validation is through discharge coefficient and surface adiabatic effectiveness, the latter of which is demonstrated to be particularly poorly modeled in RANS. Additionally, the SKE model is particularly bad at estimating the shape and strength of separation regions which strongly effect the performance of shaped holes.

More recent work in the TTCRL has shown a clear link, both computationally and experimentally, between inlet separation, inlet geometry and crossflow biasing. Using a combination of additive manufacturing and CFD we have identified and tested geometries that give us insight into how the inlet region might affect film cooling. Just from the most basic design changes we have demonstrated dramatic increases in performance and the ability to significantly change hole perfomance in crossflow.

Figure X – Even moderate 12% rounding of the inlet helped to reduce the size of the separation region in standard 7-7-7 holes and effectively reduce the flow velocity entering the diffuser. However, a separation region still persists.

Figure Y – Experiments using additively manufactured shaped holes both with and without inlet rounding show a dramatic improvement in performance. At high crossflow velocity ratios the spatially averaged effectiveness can more than double a standard 7-7-7 hole.

Figure Z – CFD showed a significant “Sweep” could significantly reduce biasing by gently redirecting the flow and reducing the strength of inlet separation caused by a sharp inlet.

Figure Also Z – Sweeping the inlet virtually eliminates jet biasing for VRi > 2.5 which prevents ingestion and increases performance relative to the standard 7-7-7.

Unfortunately, geometry limitations imposed by traditional plunge electro-discharge machining (EDM) techniques create less than ideal flows within the shaped hole. The most obvious deficiency is the sharp inlet edge created when a plunge EDM tool penetrates through a surface into the internal channel which carries the coolant. With no practical way of modifying the sharp inlet, vast amounts of research have gone into developing hole geometries that maintain a reasonable performance over a wide range of installation locations and operating conditions.

Until recently, the difficulties of in-hole measurements, expense of accurate CFD, and limitations of traditional manufacturing have driven researchers away from investigating the inlet effects on the performance of film cooling holes. By creating shaped hole inlet geometries that mitigate or suppress various deficiencies in shaped hole design we can study the effects of inlet physics on performance.

While AM still cannot produce monocrystalline blades necessary for operating in the hot end, the technology is rapidly advancing and it seems only a matter of time before the full hole geometry becomes a mutable quantity. In the following decades AM will become a mature technology and film cooling holes may look nothing like the laidback fan shapes we see today. However, computational studies show that the current generation of laidback fan shaped holes are still one of the best designs for delivering a film of protective coolant to a components surface.

Basic fluid dynamics tell us that coolant will create a separation region when forced to turn 60~90 degrees (plenum or crossflow conditions).

One of the biggest improvements to film cooling effectiveness has been the adoption of shaped holes over cylindrical holes [cite Bogard/Thole & Bunker]. A typical shaped hole is a cylindrical metering hole leading to a diffuser designed to spreads the coolant over a wider region and reduces the coolant velocity relative to the mainstream [cite Penn State 777]. When performing ideally the shaped hole should intake air from an internal cooling passage, form a uniform velocity profile entering the diffuser, reduce the coolant velocity by a factor of the area ratio, and then eject the coolant uniformly along the surface so it remains as a coherent buffer from the hot mainstream.

(generic intro that explains why people even pay us in the first place) Industry continues to demand greater performance from film cooling to allow ever higher combustion temperatures necessary for greater thermodynamic efficiency. Improvements in performance evolve from new manufacturing technologies, better experimental techniques, and more accurate simulations. One of the biggest improvements to cooling effectiveness was been the adoption of shaped holes over cylindrical holes [cite Bogard & cite Bunker]. A typical shaped hole is a cylindrical metering hole leading to a diffused outlet that spreads the coolant over a wider region and slows it relative to the mainstream [cite Penn State 777] (this sentence just feels completely thrown in here. Perhaps re-order with the last sentence?). Adding shaped holes to turbine components was made possible by advances in plunge electron discharge machining (EDM) allowing precise, repeatable hole profiles to be cut into any metallic component. (this last sentence is almost pointless. I need something better to tie this together with)

One of the biggest improvements to cooling effectiveness was been the adoption of shaped holes over cylindrical holes [cite Bogard & cite Bunker]. A typical shaped hole is a cylindrical metering hole leading to a diffused outlet that spreads the coolant over a wider region and slows it relative to the mainstream [cite Penn State 777] Yet some fundamental limitations exist simple because of manufacturing limitations and the basic design of a film cooling hole what about the basic design? Crappy sentence. Most obvious is the existence of a large separation at the metering hole inlet caused by a large change in flow angle as coolant enters the hole [use John’s cite 13] this is the whole crux of my work so why is this buried in the middle of a paragraph?. While this separation has been known to exist for decades, no effort has been made to directly study the impact of the separation (I need to do a better lit review before I stick my neck out and say this) on film cooling performance because of limitations in traditional manufacturing. However not sure if I should lead with However, recent advances in additive manufacturing technology are quickly making the entire hole geometry a mutable maybe need a better adjective quantity. While companies rush to see if computational optimization might create entirely new film cooling designs plenty of papers showing this, traditional laidback fan shaped holes are still extremely effective [cite that multi-hole CFD study that shows laidback fan to still be the best]. It makes sense then to leverage AM to study and improve existing designs in ways that had been out of reach to traditional manufacturing Does it really make sense because, as written, this seems like a real leap of logic.

[I need a picture of a standard shaped hole and also one showing the inlet separation. John has a few in his proposal I might… borrow]

(This Paragraph elaborates on part 1) To improve the existing shaped hole we must determine the causes of its poor performance in certain conditions Why should it perform well in the first place? Maybe just delete this sentence. Two of the biggest issues with shaped holes are poor use of the diffuser and jet separation at the exit of the hole [cite Bogard]. Basic fluid principles tells us that diffusers perform best and jets separate least when they receive uniform flow and changes of angle are minimized [cite that fluids guide from GE]. A simple glance at the cross-section of a standard 7-7-7 shaped hole [cite Thole] shows the inlet, outlet, and metering-to-diffuser transition region all have sharp changes in angles. The inlet is especially critical since any disturbance in the flow propagates upstream into the diffuser. Currently no literature exists explicitly investigating the effects of separation at these locations I literally said this last paragraph (redundant) but a recent experiment with additively manufactured holes at the TTCRL shows dramatic improvements in performance by adding a significant fillet at the inlet of the hole not sure if I cite myself here. This effectively increases the hydraulic diameter of the metering hole and provides a slower, more uniform flow to the diffuser inlet. It also reduces the turbulence within the Jet [cite Todd]

Scrap stuff I’ve written and thought might be useful so I’m not deleting it

To further demonstrate that we could reduce biasing of a shaped hole we created a “Swept” inlet (I’m going to need a picture of this hole to explain this design). This design effectively eliminated biasing at VRi > 2.5 and mitigates the biasing at lower VRi. While the performance at high VRc (0.3 – 0.6) and VRi (1.5>) was greatly improved, the low VRi performance was worse across the board and at VRc = 0.2 the performance never improved relative to the on-design hole. Also of interest was the fact that a simple rounding of the inlet matched or outperformed the swept hole in almost all cases. Investigation of lateral profiles and surface contours show that the biasing spreads out the coolant jets across the surface resulting in a net increase in coverage, although this comes at the cost of potential ingestion.